Table 1. Neural Recording Modality Comparison Matrix — Systematic Review of AI for Brain-to-Speech Decoding (2015–2026)

SNR = signal-to-noise ratio. WER = word error rate. PCC = Pearson correlation coefficient. WPM = words per minute. CER = character error rate. BLEU = bilingual evaluation understudy score. Performance ranges reflect best-reported values within this review's 71 included studies. Group A invasive studies (n=32) are subdivided by electrode type: ECoG-family n=22, intracortical n=8, sEEG n=4. Study counts per modality row are approximate where a study uses more than one modality type. Total n=71 across all five groups.

| Modality | Invasiveness | Spatial resolution | Temporal resolution | Signal SNR | Participant burden | Representative performance range (this review) | Studies (n) | Primary decoding task |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GROUP A — INVASIVE MODALITIES (n = 32 studies) |  |  |  |  |  |  |  |  |
| ECoG (standard / HD-ECoG / µECoG) | Craniotomy required; subdural grid or strip placement on cortical surface | ~1–5 mm between electrodes ~1 mm² cortical coverage per contact | ~1 ms; broadband high-gamma (70–150 Hz) resolves speech transients | High Minimal skull attenuation; low noise floor | Surgical implantation; typically epilepsy or ALS clinical populations | WER: 3–26% (overt / attempted speech) PCC: 0.69–0.87 (acoustic reconstruction) Accuracy: 58–97% (phoneme / word classification) Key studies: Makin 2020 WER 3%; Moses 2021 WER 26%; Chen 2024 PCC 0.804 (n=48); Metzger 2023 WER 25% | 22 | Overt, attempted, and imagined speech; phoneme and word classification; acoustic reconstruction |
| Intracortical (Utah array / microelectrode) | Neurosurgical implantation of microelectrode arrays into cortical grey matter | ~400 µm between electrodes Single-neuron and multi-unit activity resolution | <1 ms; single-spike temporal resolution; finest of all modalities | Very high Direct cortical access; spike-level SNR | Permanent or semi-permanent implant; primarily ALS / paralysis patient populations | WER: 2.5–9% (attempted speech, open vocabulary) WPM: 32–78 words per minute Longitudinal: 97.5% accuracy sustained >8 months (Card 2024) Key studies: Card 2024 WER 2.5%; Willett 2023 WER 9% / 62 WPM; Metzger 2023 78 WPM | 8 | Attempted / silent speech; open-vocabulary text and voice synthesis; real-time neuroprosthesis |
| sEEG (stereotactic / depth electrodes) | Minimally invasive depth electrode placement; no craniotomy required | ~3.5 mm between contacts Samples deep cortical and subcortical regions | ~1 ms; lower high-gamma sensitivity than ECoG; broader regional coverage | Moderate–high Less tissue displacement than ECoG grids | Epilepsy monitoring populations; increasing ALS and speech neuroprosthesis use | Accuracy: Group decoder outperforms individual decoders (Singh 2025) Audio PCC: Comparable to ECoG (Kohler 2022) CER: ~29% logosyllabic Chinese (Feng 2023) Key studies: Singh 2025 (n=25 sEEG, cross-participant); Angrick 2021 real-time imagined speech | 4 | Overt and imagined speech; cross-participant transfer; phoneme and syllable classification |
| GROUP B — NON-INVASIVE MODALITIES (n = 31 studies across EEG / MEG / fMRI) |  |  |  |  |  |  |  |  |
| Scalp EEG (electroencephalography) | None; surface scalp electrode placement via cap-based systems | ~10–50 mm (scalp level) Severe spatial smearing via skull and scalp tissue | ~1 ms; excellent temporal resolution but very low spatial specificity | Low Significant skull / scalp attenuation; artifact-prone | No surgery; suitable for healthy participants and outpatient clinical use | Closed-set accuracy: 70–92.5% (imagined speech classification) Open-vocabulary WER: ~35–45% (EEG-to-text paradigm) SSL gain: +5–15% over supervised baselines (BENDR, Kostas 2021) Key: Yang 2024 WER 35–45%; Agarwal 2022 73.56%; Abdulghani 2023 92.5%; Défossez 2023 (n=175) | 20 | Imagined / covert speech classification; perceived speech decoding; open-vocabulary text generation |
| MEG (magnetoencephalography) | None; superconducting SQUID sensor arrays; large fixed gantry system required | ~5–10 mm; better than EEG Tangential source sensitivity; poor radial sensitivity | ~1 ms; comparable to EEG; superior to fMRI | Moderate Requires magnetically shielded room; sensitive to motion | No surgery; expensive specialized facility required; limits ecological validity | Top-1 accuracy: 41% (Défossez 2023, n=175); up to 60.6% (d'Ascoli 2025, n=723, 3 languages) Mel-PCC: ~0.95 (Kwon 2024 — first intelligible MEG speech synthesis) BERTScore: 0.816 (Wang et al. MEG semantic reconstruction) Key: d'Ascoli 2025 (723 participants, 7 datasets, 3 languages); Kwon 2024 BigVGAN synthesis | 5 | Perceived speech decoding; word classification; acoustic synthesis; semantic reconstruction |
| fMRI (functional MRI) | None; BOLD signal acquisition in MRI scanner; no implant or preparation | ~1–3 mm voxel Highest spatial resolution of all non-invasive modalities | ~1–2 s (TR) Severe temporal limitation; hemodynamic response lags neural activity by ~5 s | Moderate BOLD is indirect; excellent spatial SNR; sensitive to head motion | No surgery; MRI scanner required; immobile during acquisition; high facility cost | Story identification: 100% (Tang 2023, n=3) BLEU-1: 27.8% (Yin 2024 brain predictive coding) Cross-participant: Demonstrated (Zhang 2024; Tang & Huth 2025) Key: Tang 2023 semantic reconstruction; Tang & Huth 2025 cross-participant multi-modal; Qiu 2025 MindLLM | 6 | Perceived and imagined language; continuous semantic reconstruction; LLM-integrated decoding |
| GROUP D — HYBRID AND PERIPHERAL MODALITIES (n = 6 studies) |  |  |  |  |  |  |  |  |
| HD-sEMG / Hybrid (EEG+fNIRS, HD-sEMG, sEMG) | None for sEMG and EEG+fNIRS; surface electrode placement only | Variable: HD-sEMG ~few mm (muscle surface) EEG+fNIRS: scalp-level spatial resolution | ~1 ms (sEMG) ~100–200 ms (fNIRS); combined modalities offer complementary temporal coverage | High (sEMG) Peripheral signal strong; fNIRS adds hemodynamic context to EEG | No surgery; wearable systems feasible; HD electrode arrays require careful placement | HD-sEMG phrase accuracy: 97.17% ±1.53% (Song 2023, n=15) CER: 3.11% ±1.46% (Song 2023 — 33 Chinese phrases) EEG+fNIRS gain: +8–12% over EEG alone (Sun 2024; Cooney 2022) Key: Song 2023 best CER in silent speech; Zhu 2021 87–90% (English/Chinese digits); Li 2022 Mandarin tonal | 6 | Silent speech; articulatory-based decoding; imagined speech classification; tonal language synthesis |
| GROUP E — FOUNDATION MODELS / TRANSFER LEARNING APPROACHES (n = 2 studies) |  |  |  |  |  |  |  |  |
| Foundation model / SSL (cross-modal approach) | Spans invasive modalities (iEEG, ECoG); pre-trained on large unlabeled neural corpora | Inherited from source modality (iEEG or ECoG) Not modality-specific | Inherited from source modality SSL improves effective SNR via large-scale pre-training | Inherited from source modality SSL pre-training improves effective SNR by learning signal structure | Requires large unlabeled neural datasets; few-shot fine-tuning minimizes participant burden | Downstream accuracy: Improved over supervised baselines (BrainBERT, Wang 2023) Reduced WER: SSL pretraining reduces WER on ECoG tasks (Yuan & Makin 2024) SSL approaches treat brain signal pre-training as analogous to wav2vec / BERT for speech and language Key: Wang 2023 BrainBERT (ICLR 2023, iEEG SSL foundation model); Yuan & Makin 2024 (ECoG SSL) | 2 | Downstream speech classification; WER reduction via pre-trained neural representations; cross-participant transfer |

Notes. Representative performance ranges are derived from included studies in this review only and should not be interpreted as field-wide maxima. Participant burden ratings reflect typical clinical and research practice for each modality. Foundation model studies (Group E) inherit their technical characteristics from the source invasive modality used during pre-training. Studies using multimodal approaches (e.g., EEG+fNIRS) are classified under the hybrid/peripheral group (Group D). The n values for Groups B, C, D, and E reflect primary modality groupings as defined in the registered protocol.